Physics-Informed Neural Networks (PINNs)
for DC Magnetron Sputtering

A Complete Implementation Guide for the SputterTwin Digital Twin Project

Target Material: Copper (Cu)  |  Framework: Python + PyTorch

September 2026

# Table of Contents

1. Executive Summary

2. What Are PINNs and Why Use Them for Sputtering?

3. Our Current SputterTwin Project — What We Already Have

4. The Physics Governing Equations for DC Magnetron Sputtering

5. PINN Architecture Design for Each Sub-Process

6. Step 1: Plasma Discharge PINN

7. Step 2: Sputter Yield PINN

8. Step 3: Gas Transport PINN

9. Step 4: Film Deposition & Thickness Profile PINN

10. Step 5: Combining All PINNs — The Coupled Digital Twin

11. Step 6: Training Strategy & Loss Function Design

12. Step 7: Testing & Validation

13. Step 8: Running Experiments & Collecting Data

14. Can the Same Model Work for Different Target Materials?

15. Implementation Roadmap & Timeline

16. Appendix: Code Templates

# 1. Executive Summary

This document provides a complete, step-by-step guide for implementing Physics-Informed Neural Networks (PINNs) within our existing SputterTwin digital twin project for DC magnetron sputtering. The guide is tailored for a Copper (Cu) target material and covers:

What PINNs are and how they differ from our current analytical models

How to encode each physics sub-process (plasma, sputtering, transport, deposition) as a PINN

How to combine individual PINNs into a coupled system

How to train, test, validate, and run real experiments

Whether the same model can handle different target materials (short answer: yes, with material-conditioned architectures or transfer learning)

[IMPORTANT] PINNs do NOT replace our existing physics equations — they augment them. Our current analytical models in plasma.py, sputter_yield.py, transport.py, and deposition.py become the physics loss terms that constrain the neural networks during training.

# 2. What Are PINNs and Why Use Them for Sputtering?

## 2.1 Standard Neural Network vs PINN

A standard neural network learns purely from data:

Loss = L_Data = MSE(y_predicted, y_measured)

A Physics-Informed Neural Network adds physics constraints:

Loss = λ₁·L_Data + λ₂·L_Physics + λ₃·L_BoundaryConditions + λ₄·L_InitialConditions

Where:

L_Data — error between network predictions and experimental measurements

L_Physics — residual of the governing differential equations (PDEs/ODEs) evaluated at collocation points using automatic differentiation

L_Boundary — error at known boundary conditions

L_Initial — error at initial conditions

## 2.2 Why PINNs for DC Magnetron Sputtering?

| Challenge in Sputtering | How PINNs Help |
| --- | --- |
| Sparse experimental data — film thickness measurements are expensive | PINNs need far less data because physics equations act as regularizers |
| Multi-physics coupling — plasma, sputtering, transport, deposition are all coupled | PINNs can embed multiple coupled PDEs in a single loss function |
| Unknown parameters — γ_SE, sticking coefficient, etc. are poorly known | PINNs can solve inverse problems to discover these parameters from data |
| Real-time digital twin — traditional FEM/PIC simulations take hours | Trained PINNs predict in milliseconds |
| Process optimization — finding optimal (Power, Pressure, Distance) | PINNs enable rapid gradient-based optimization over the entire parameter space |

## 2.3 Comparison with Our Current Approach

| Aspect | Current SputterTwin (Analytical) | PINNs-Enhanced SputterTwin |
| --- | --- | --- |
| Model type | Closed-form empirical equations | Neural network constrained by those same equations |
| Calibration | Manual tuning of constants | Automatic learning from data while obeying physics |
| Accuracy | Good at nominal, degrades at extremes | Learns corrections from data, valid across wider range |
| Speed | Instantaneous | ~1-5 ms per prediction (after training) |
| Inverse problems | Not possible | Can estimate unknown γ_SE, sticking coeff., etc. |
| Extrapolation | Poor (empirical fits break down) | Better — physics loss prevents unphysical predictions |

# 3. Our Current SputterTwin Project — What We Already Have

Our existing codebase provides the perfect physics backbone for PINNs. Here's what each module contributes:

## 3.1 Project Structure

sputter_twin/

├── sputtertwin/

│   ├── physics/

│   │   ├── plasma.py          → DC discharge I-V-P model (magnetron power-law)

│   │   ├── sputter_yield.py   → Yamamura-Tawara yield model (Cu, Ti, Al)

│   │   ├── transport.py       → Gas kinetic theory (MFP, Kn, Beer-Lambert)

│   │   └── deposition.py      → Film thickness distribution & uniformity

│   └── __init__.py

├── tests/test_physics.py      → 17 comprehensive physics tests

├── scripts/generate_plots.py  → Visualization (5 diagnostic plots)

└── requirements.txt

## 3.2 Existing Physics Models (These Become Our PINN Loss Terms)

### Plasma Discharge (plasma.py)

Governing relation: I_d = k · P_eff^m · V_d^n (magnetron power-law)

Key outputs: Voltage (V_d), Current (I_d), Ion flux (Γ_i), Electron temperature (T_e), Plasma density (n_e)

Material for Cu: γ_SE = 0.10 (secondary electron emission for Ar⁺ on Cu)

### Sputter Yield (sputter_yield.py)

Governing model: Yamamura-Tawara / Bohdansky formulation

Cu parameters already defined: TargetMaterial("Cu", 63.55, 29, 8.96, 3.49, 20.0, 1.60)

→ Atomic mass: 63.55 g/mol, Z=29, ρ=8.96 g/cm³

→ Sublimation energy: 3.49 eV, Threshold: 20.0 eV, Q-factor: 1.60

Angular dependency: Yamamura Y(θ) = Y(0)·cos(θ)^(-f)·exp(-Σ·(sec(θ)-1))

### Gas Transport (transport.py)

Mean free path: λ = k_B·T / (P·σ) (kinetic theory)

Transmission probability: P_bal = exp(-d/λ) (Beer-Lambert)

Scattering broadening: BF = sqrt(1 + d/λ) (random-walk)

### Film Deposition (deposition.py)

Rate model: R = (I_ion/e)·Y·η_transport·η_chamber / (π·d²)·Ω

Profile model: Parabolic h(r) = t₀·(1 - c_eff·r²/R²)

Uniformity: NU = (max-min)/(2·mean) × 100%

# 4. The Physics Governing Equations for DC Magnetron Sputtering

Below are the governing equations that will form the physics loss terms in our PINNs, specifically calibrated for Copper (Cu) target with Ar⁺ bombardment.

## 4.1 Plasma Discharge Physics

Magnetron I-V Characteristic (Power-Law):

I_d = k · P_eff^m · V_d^n

where k = geometry/confinement constant, P_eff = effective pressure (mTorr), V_d = discharge voltage (V, typically 300-500 V), n = 6.0, m = 0.4

Power conservation (hard constraint — must be satisfied exactly):

W = V_d · I_d

Sheath ion energy:

E_ion ≈ V_d

Ion current (accounting for secondary electrons):

I_ion = (1 - γ_SE) · I_d     [For Cu: γ_SE ≈ 0.10]

Electron temperature (empirical scaling):

T_e = 3.0 · (5.0 / P_eff)^0.25  eV, bounded [2.0, 4.0]

Plasma density (Bohm sheath criterion):

Γ_i = exp(-0.5) · n_e · sqrt(e · T_e / M_Ar)

## 4.2 Sputter Yield Physics (Cu Target)

Yamamura-Tawara Formulation:

Y(E, 0) = (0.042 / U_s) · α · Q · S_n(E) · [1 - sqrt(E_th / E)]^2.8

For Cu: U_s = 3.49 eV (sublimation energy), E_th = 20.0 eV (threshold energy), Q = 1.60 (empirical scaling factor), α = f(M_Cu/M_Ar) ≈ 0.10 + 0.155 × (63.55/39.948)^0.73

Angular dependency:

Y(E, θ) = Y(E, 0) · cos(θ)^(-f) · exp[-Σ · (1/cos(θ) - 1)]

## 4.3 Gas Transport Physics

Mean free path (Ar background gas):

λ = k_B · T / (P · σ)     [σ ≈ 4.0 × 10⁻¹⁹ m² for Cu–Ar]

Ballistic transmission (Beer-Lambert):

P_ballistic = exp(-d / λ)

Transport efficiency (thermalization model):

η_transport = 1 / (1 + n_coll / N_therm)     [n_coll = d/λ, N_therm = 10]

## 4.4 Film Deposition & Growth

Deposition rate:

R_dep = (I_ion · Y / e) · η_transport · η_chamber · Ω / (π · d²)

where Ω = M_Cu / (ρ_Cu · N_A) is the atomic volume of Cu

Radial profile:

h(r) = t₀ · (1 - c_eff · r² / R_wafer²)

c_eff = γ_profile · (r_race / d)² / BF²

# 5. PINN Architecture Design for Each Sub-Process

## 5.1 Overall Architecture Strategy

We will build 4 individual PINN modules that mirror our existing physics modules, then combine them into a coupled pipeline:

┌─────────────┐    ┌──────────────┐    ┌───────────────┐    ┌────────────────┐

│ Plasma PINN │───►│ Yield PINN   │───►│Transport PINN │───►│Deposition PINN │

│             │    │              │    │               │    │                │

│ Inputs:     │    │ Inputs:      │    │ Inputs:       │    │ Inputs:        │

│  W, P, flow │    │  E_ion       │    │  P, d, T      │    │  All upstream  │

│             │    │  material    │    │               │    │  + geometry    │

│ Outputs:    │    │ Output:      │    │ Outputs:      │    │ Outputs:       │

│  Vd,Id,Γi  │    │  Y(E,θ)     │    │  λ,Kn,η      │    │  h(r), NU, R  │

│  Te,ne     │    │              │    │               │    │                │

└─────────────┘    └──────────────┘    └───────────────┘    └────────────────┘

## 5.2 Recommended Hyperparameters per Sub-Module

| Sub-Module | Input Dim | Hidden Layers | Neurons/Layer | Activation | Output Dim |
| --- | --- | --- | --- | --- | --- |
| Plasma PINN | 3 (W, P, flow) | 4 | 64 | tanh | 5 (Vd, Id, Γi, Te, ne) |
| Yield PINN | 2 (E, θ) | 3 | 32 | tanh | 1 (Y) |
| Transport PINN | 3 (P, d, T) | 3 | 32 | tanh | 3 (λ, Kn, η) |
| Deposition PINN | 6 (W, P, d, t, flow, r) | 5 | 128 | tanh | 2 (h, rate) |

## 5.3 Why tanh Activation?

PINNs require computing derivatives of the network output with respect to inputs (via automatic differentiation)

tanh is smooth and infinitely differentiable — ideal for PDE residual computation

ReLU has discontinuous derivatives → problematic for PINNs

Alternative: sin activations (SIREN networks) for problems with high-frequency components

# 6. Step 1: Plasma Discharge PINN

## 6.1 Goal

Learn the mapping: (Power W, Pressure P, Ar Flow) → (V_d, I_d, Γ_i, T_e, n_e) while enforcing the magnetron power-law and energy conservation.

## 6.2 Network Architecture

class PlasmaPINN(nn.Module):

def __init__(self, hidden_layers=4, neurons=64):

super().__init__()

layers = [nn.Linear(3, neurons), nn.Tanh()]  # Input: [W, P, flow]

for _ in range(hidden_layers - 1):

layers += [nn.Linear(neurons, neurons), nn.Tanh()]

layers += [nn.Linear(neurons, 5)]  # Output: [Vd, Id, Γi, Te, ne]

self.net = nn.Sequential(*layers)

def forward(self, x):

raw = self.net(x)

Vd = F.softplus(raw[:, 0])    # Voltage must be positive

Id = F.softplus(raw[:, 1])    # Current must be positive

Gi = F.softplus(raw[:, 2])    # Ion flux must be positive

Te = 2.0 + 2.0 * torch.sigmoid(raw[:, 3])  # Te bounded [2, 4] eV

ne = F.softplus(raw[:, 4])    # Density must be positive

return torch.stack([Vd, Id, Gi, Te, ne], dim=1)

## 6.3 Physics Loss Terms

def plasma_physics_loss(model, W, P_eff):

outputs = model(inputs)

Vd, Id, Gi, Te, ne = outputs[:, 0], outputs[:, 1], outputs[:, 2], outputs[:, 3], outputs[:, 4]

# LOSS 1: Power conservation (hard constraint)

L_power = torch.mean((W - Vd * Id) ** 2)

# LOSS 2: Magnetron I-V characteristic

Id_predicted = K_DISCHARGE * (P_eff ** M_EXP) * (Vd ** N_EXP)

L_IV = torch.mean((Id - Id_predicted) ** 2)

# LOSS 3: Ion current relation

I_ion = (1.0 - GAMMA_SE) * Id

Gi_predicted = I_ion / (ELEMENTARY_CHARGE * A_RACE)

L_flux = torch.mean((Gi - Gi_predicted) ** 2)

# LOSS 4: Bohm sheath criterion

u_bohm = torch.sqrt(ELEMENTARY_CHARGE * Te / M_AR_KG)

ne_predicted = Gi / (math.exp(-0.5) * u_bohm)

L_bohm = torch.mean((ne - ne_predicted) ** 2)

return L_power + L_IV + L_flux + L_bohm

## 6.4 Training Data Sources

| Data Source | What It Provides | How to Get It |
| --- | --- | --- |
| Our analytical model | Synthetic training data at any (W, P, flow) | Run calculate_discharge_state() for 1000+ points |
| Experimental I-V curves | Real Vd, Id at known W, P | Measure with power supply readback |
| Langmuir probe data | Te, ne at specific conditions | Langmuir probe in chamber (if available) |
| Literature values | Reference Vd, Id ranges for Cu targets | Published Cu magnetron discharge data |

## 6.5 How to Start (Step-by-Step)

1. Generate synthetic training data from our existing calculate_discharge_state() — sample 2000 points across W=[50,500]W, P=[1,20]mTorr

2. Split into training (80%) / validation (10%) / test (10%)

3. Add 2% Gaussian noise to training data to simulate real experimental scatter

4. Train with combined loss: total_loss = λ_data × L_data + λ_physics × L_physics. Start with λ_data=1.0, λ_physics=0.1, then gradually increase λ_physics

5. Validate against clean analytical model and (later) experimental data

# 7. Step 2: Sputter Yield PINN

## 7.1 Goal

Learn: (Ion Energy E, Angle θ) → Sputter Yield Y while enforcing: Y=0 below threshold (E < 20 eV for Cu), Y monotonically increases with E, Y=0 at grazing angles (θ > 85°), and the Yamamura-Tawara functional form.

## 7.2 Network Architecture

class SputterYieldPINN(nn.Module):

def __init__(self, hidden_layers=3, neurons=32):

super().__init__()

layers = [nn.Linear(2, neurons), nn.Tanh()]  # Input: [E, θ]

for _ in range(hidden_layers - 1):

layers += [nn.Linear(neurons, neurons), nn.Tanh()]

layers += [nn.Linear(neurons, 1)]  # Output: Y

self.net = nn.Sequential(*layers)

def forward(self, E, theta):

x = torch.stack([E, theta], dim=1)

Y_raw = self.net(x).squeeze()

Y = F.softplus(Y_raw)

mask = (E > E_TH_CU).float()  # E_TH_CU = 20.0 eV for Cu

return Y * mask

## 7.3 Cu-Specific Literature Validation Points

| Energy (eV) | Y (atoms/ion) — Literature | Source |
| --- | --- | --- |
| 200 | 1.0 – 1.5 | Yamamura & Tawara (1996) |
| 400 | 2.0 – 2.8 | Behrisch & Eckstein (2007) |
| 600 | 2.5 – 3.5 | SRIM simulation |
| 1000 | 3.0 – 4.5 | Various experimental |

[NOTE] Cu has a higher sputter yield than Ti because of its lower sublimation energy (3.49 eV vs 4.89 eV) and higher Q-factor (1.60 vs 0.85). At 400 eV, Y_Cu ≈ 2.3 vs Y_Ti ≈ 0.55.

# 8. Step 3: Gas Transport PINN

## 8.1 Goal

Learn: (Pressure P, Distance d, Temperature T) → (λ, Kn, η_transport, BF) while enforcing kinetic theory and Beer-Lambert law.

## 8.2 Why PINN Here Instead of Just Analytical?

For transport, the analytical model is very accurate. The PINN's value here is:

Differentiability — enables end-to-end gradient flow when coupled with other PINNs

Learning corrections — real transport has energy-dependent cross-sections not captured by simple kinetic theory

Inverse problems — estimating the actual collision cross-section σ from experimental deposition rate data

# 9. Step 4: Film Deposition & Thickness Profile PINN

## 9.1 Goal

This is the most important PINN — it predicts the final measurable quantity: (W, P, d, t, flow, r_wafer) → Film Thickness h(r)

## 9.2 Experimental Data Needed for Cu

| Measurement | How to Obtain | Typical Accuracy |
| --- | --- | --- |
| Mean thickness | Profilometer (stylus or optical) | ±1 nm |
| Radial profile | Multi-point profilometry across wafer | ±2 nm |
| Deposition rate | thickness / time | ±5% |
| Uniformity | (max-min)/(2×mean) × 100% | ±0.1% |
| Sheet resistance | 4-point probe → thickness via ρ_Cu | ±3% |

# 10. Step 5: Combining All PINNs — The Coupled Digital Twin

## 10.1 Architecture: Cascaded PINN Pipeline

class SputterTwinPINN(nn.Module):

"""Complete Physics-Informed Digital Twin for DC Magnetron Sputtering."""

def __init__(self):

super().__init__()

self.plasma_pinn = PlasmaPINN()

self.yield_pinn = SputterYieldPINN()

self.transport_pinn = TransportPINN()

self.deposition_pinn = DepositionPINN()

def forward(self, W, P, flow, d, t, r):

# Stage 1: Plasma discharge

plasma_out = self.plasma_pinn(torch.stack([W, P, flow], dim=1))

Vd, Id, Gi, Te, ne = [plasma_out[:, i] for i in range(5)]

# Stage 2: Sputter yield (using ion energy from plasma)

E_ion = Vd

Y = self.yield_pinn(E_ion, torch.zeros_like(E_ion))

# Stage 3: Gas transport

transport_out = self.transport_pinn(torch.stack([P, d, T_gas], dim=1))

lam, Kn, eta, BF = [transport_out[:, i] for i in range(4)]

# Stage 4: Film deposition

h = self.deposition_pinn(torch.stack([W, P, d, t, flow, r], dim=1))

return {'thickness_nm': h, 'voltage_v': Vd, 'current_a': Id,

'sputter_yield': Y, 'transport_efficiency': eta}

## 10.2 Coupling Strategy Options

| Strategy | Description | When to Use |
| --- | --- | --- |
| End-to-End | Train all 4 PINNs simultaneously as one model | When you have full pipeline data |
| Sequential | Train plasma → yield → transport → deposition one at a time | When starting out; easier to debug |
| Hybrid (Recommended) | Pre-train each sub-PINN on synthetic data, then fine-tune end-to-end on experimental data | Best of both worlds |

# 11. Step 6: Training Strategy & Loss Function Design

## 11.1 Training Pipeline

Phase 1: Pre-training on Synthetic Data (from analytical models)

Generate 10,000+ points from calculate_discharge_state()

Generate 5,000+ points from calculate_sputter_yield()

Generate 5,000+ points from calculate_transport_summary()

Generate 10,000+ points from simulate_deposition()

Train each sub-PINN independently for 5,000 epochs

Phase 2: Physics-Informed Training (enforce PDEs)

Add physics loss terms with increasing weight

Use collocation points sampled across the domain

Train for 10,000 epochs with ADAM optimizer

Switch to L-BFGS for final fine-tuning (1,000 iterations)

Phase 3: Experimental Data Fusion (when available)

Incorporate real measurement data

Fine-tune with combined data + physics loss

Use self-adaptive loss balancing

## 11.2 Optimizer Strategy

# Phase 1-2: ADAM with learning rate scheduling

optimizer = torch.optim.Adam(model.parameters(), lr=1e-3)

scheduler = torch.optim.lr_scheduler.ReduceLROnPlateau(

optimizer, patience=500, factor=0.5, min_lr=1e-6

)

# Phase 3: Switch to L-BFGS for fine-tuning

optimizer_lbfgs = torch.optim.LBFGS(

model.parameters(), lr=1.0, max_iter=20,

history_size=50, line_search_fn='strong_wolfe'

)

## 11.3 Self-Adaptive Loss Balancing

The different loss terms can have vastly different magnitudes. Use self-adaptive weighting with learnable log-variance parameters:

class AdaptiveLossWeights(nn.Module):

def __init__(self, n_losses):

super().__init__()

self.log_vars = nn.Parameter(torch.zeros(n_losses))

def forward(self, losses):

total = 0

for i, loss in enumerate(losses):

precision = torch.exp(-self.log_vars[i])

total += precision * loss + self.log_vars[i]

return total

## 11.4 Collocation Point Sampling

def sample_collocation_points(n_points=5000):

W = torch.FloatTensor(n_points).uniform_(50, 500)       # 50-500 W

P = torch.FloatTensor(n_points).uniform_(1, 20)          # 1-20 mTorr

d = torch.FloatTensor(n_points).uniform_(40, 150)         # 40-150 mm

t = torch.FloatTensor(n_points).uniform_(60, 3600)        # 1-60 min

flow = torch.FloatTensor(n_points).uniform_(5, 50)        # 5-50 sccm

r = torch.FloatTensor(n_points).uniform_(0, 75)           # 0-75 mm

return W, P, d, t, flow, r

# 12. Step 7: Testing & Validation

## 12.1 Validation Hierarchy

Level 1: Internal Consistency

Power conservation: W = Vd × Id (must be exact)

Yield threshold: Y(E < 20 eV) = 0 for Cu

Zero power → zero thickness

Thickness linearity with time

Level 2: Analytical Model Agreement

Compare PINN predictions with calculate_discharge_state()

Compare with calculate_sputter_yield('Cu', ...)

Compare with simulate_deposition(material='Cu', ...)

Target: < 2% relative error across operating space

Level 3: Experimental Validation

Compare with measured film thickness (profilometry)

Compare with measured I-V curves

Compare with measured deposition rates

Target: < 10% relative error

Level 4: Extrapolation Testing

Predict at conditions outside training data

Compare with experimental measurements at extreme conditions

Verify physics constraints are maintained

# 13. Step 8: Running Experiments & Collecting Data

## 13.1 Phase A: Baseline Characterization (10-15 runs)

| Run | Power (W) | Pressure (mTorr) | Distance (mm) | Time (s) | Purpose |
| --- | --- | --- | --- | --- | --- |
| 1 | 220 | 5.0 | 80 | 720 | Nominal baseline |
| 2 | 220 | 5.0 | 80 | 360 | Half time (linearity check) |
| 3 | 220 | 5.0 | 80 | 1440 | Double time (linearity check) |
| 4 | 100 | 5.0 | 80 | 720 | Low power |
| 5 | 400 | 5.0 | 80 | 720 | High power |
| 6 | 220 | 2.0 | 80 | 720 | Low pressure |
| 7 | 220 | 10.0 | 80 | 720 | High pressure |
| 8 | 220 | 20.0 | 80 | 720 | Very high pressure |
| 9 | 220 | 5.0 | 50 | 720 | Short distance |
| 10 | 220 | 5.0 | 120 | 720 | Long distance |
| 11 | 100 | 2.0 | 50 | 720 | Corner: low P, low d |
| 12 | 400 | 15.0 | 120 | 720 | Corner: high P, high d |

## 13.2 Phase B: Multi-Variate DOE (15-20 runs)

Use Latin Hypercube Sampling to efficiently cover the parameter space (Power: 100-400 W, Pressure: 1-20 mTorr, Distance: 50-120 mm).

## 13.3 Phase C: Validation Holdout (5-8 runs)

Reserve these for pure testing — never used in training:

| Run | Power (W) | Pressure (mTorr) | Distance (mm) | Purpose |
| --- | --- | --- | --- | --- |
| V1 | 180 | 7.5 | 90 | Interior point |
| V2 | 350 | 3.5 | 65 | High power, low pressure |
| V3 | 130 | 12.0 | 100 | Low power, high pressure |
| V4 | 280 | 5.0 | 80 | Near nominal |
| V5 | 220 | 5.0 | 80 | Repeat of baseline (reproducibility) |

## 13.4 What to Measure in Each Run

| Measurement | Equipment | When | Data Points |
| --- | --- | --- | --- |
| Discharge voltage (V) | Power supply readback | During deposition | Continuous log |
| Discharge current (A) | Power supply readback | During deposition | Continuous log |
| Chamber pressure (mTorr) | Capacitance manometer | During deposition | Continuous log |
| Ar flow (sccm) | Mass flow controller | Set point | 1 value |
| Deposition time (s) | Timer | After deposition | 1 value |
| Film thickness (nm) | Profilometer | Post-deposition | 5-9 points across wafer |
| Sheet resistance (Ω/□) | 4-point probe | Post-deposition | 5-9 points across wafer |
| Film stress (MPa) | Wafer curvature | Post-deposition | 1 value (optional) |

## 13.5 Data Format

Store experimental data in CSV format:

run_id,power_w,pressure_mtorr,ar_flow_sccm,distance_mm,time_s,r_mm,thickness_nm,Vd_v,Id_a,sheet_resistance_ohm_sq

CU001,220,5.0,20.0,80,720,0,95.2,398,0.553,0.175

CU001,220,5.0,20.0,80,720,25,94.8,398,0.553,0.176

CU001,220,5.0,20.0,80,720,50,93.1,398,0.553,0.179

CU001,220,5.0,20.0,80,720,75,90.5,398,0.553,0.184

# 14. Can the Same Model Work for Different Target Materials?

## 14.1 Short Answer

A single PINN architecture CAN handle multiple target materials, but it requires one of the following approaches:

## 14.2 Approach 1: Separate Models Per Material (Simplest)

Train a completely separate PINN for each material (Cu_PINN, Ti_PINN, Al_PINN).

Pros: Simple, no interference between materials

Cons: No knowledge sharing, need lots of data per material, redundant training

## 14.3 Approach 2: Material-Conditioned Single Model (Recommended) ✅

Add material properties as input features to the network. The input now includes 6 process parameters + 6 material parameters (M, Z, ρ, Us, Eth, Q) = 12 inputs total.

MATERIAL_FEATURES = {

'Cu': [63.55, 29, 8.96, 3.49, 20.0, 1.60],  # M, Z, ρ, Us, Eth, Q

'Ti': [47.87, 22, 4.506, 4.89, 30.0, 0.85],

'Al': [26.98, 13, 2.70, 3.39, 25.0, 0.90],

}

Why this works: The physics equations (Yamamura-Tawara yield, atomic volume, etc.) are explicit functions of these material properties. By providing them as inputs, the PINN can learn the material-dependent physics from the equations themselves.

Pros: One model handles all materials, learns shared physics, works for new materials with known properties

Cons: Needs training data from multiple materials, slightly more complex

## 14.4 Approach 3: Transfer Learning (Best for Limited Data)

Train on one material (e.g., Cu with lots of data), then transfer to another material. Freeze early layers (shared physics), fine-tune later layers (material-specific) with a lower learning rate.

Pros: Needs very little data for new materials, fast adaptation

Cons: Quality depends on similarity between source and target materials

## 14.5 What Changes Between Materials?

| Component | Material-Dependent? | What Changes |
| --- | --- | --- |
| Plasma discharge | Partially — via γ_SE | γ_SE varies: Cu≈0.10, Ti≈0.07, Al≈0.09 |
| Sputter yield | Strongly | M₂, Z₂, Us, Eth, Q all change → completely different Y(E) |
| Gas transport | Partially — via σ | Collision cross-section σ depends on sputtered atom species |
| Deposition rate | Strongly | Atomic volume Ω = M/(ρ·N_A) varies significantly |
| Film properties | Completely | Stress, resistivity, grain structure all material-specific |

[IMPORTANT] Recommendation: Start with a Cu-only PINN (Approach 1). Once working and validated, extend to the Material-Conditioned approach (Approach 2) by adding material properties as inputs. The physics equations in our existing codebase already parameterize everything by material (TargetMaterial dataclass), so the transition will be natural.

# 15. Implementation Roadmap & Timeline

## Phase 1: Foundation (Weeks 1-2)

Set up PyTorch environment and dependencies

Create sputtertwin/pinn/ module directory

Implement PlasmaPINN with physics loss from plasma.py

Implement SputterYieldPINN with Yamamura-Tawara loss

Generate synthetic training data from existing analytical models

Train and validate each sub-PINN independently

## Phase 2: Integration (Weeks 3-4)

Implement TransportPINN with kinetic theory loss

Implement DepositionPINN with full pipeline loss

Build SputterTwinPINN coupled model

Implement combined loss function with adaptive weighting

Train coupled model on synthetic data

Validate against existing test suite (17 tests)

## Phase 3: Experimental Validation (Weeks 5-8)

Design experimental matrix for Cu sputtering

Run Phase A baseline experiments (10-15 runs)

Create data loading pipeline

Fine-tune PINN on experimental data

Run Phase B DOE experiments (15-20 runs)

Validate on Phase C holdout set

## Phase 4: Advanced Features (Weeks 9-12)

Implement inverse problem mode (estimate unknown γ_SE, σ, etc.)

Add material-conditioned inputs for multi-material support

Transfer learning from Cu to Ti and Al

Build real-time prediction API

Integrate with process optimization loop

Documentation and publication

## Required Dependencies

# Add to requirements.txt:

torch>=2.0

matplotlib>=3.5

pandas>=1.5

scipy>=1.9

tensorboard>=2.12  # For training visualization

# 16. Appendix: Complete Minimal PINN Example (Ready to Run)

Below is a complete, self-contained, runnable script for a minimal Plasma Discharge PINN that uses our existing analytical model as the physics backbone.

"""

Minimal working example: Plasma Discharge PINN for Cu DC Magnetron Sputtering.

Uses our existing analytical model as the physics backbone.

"""

import torch

import torch.nn as nn

import torch.nn.functional as F

import numpy as np

import sys, os

sys.path.insert(0, os.path.abspath(os.path.join(os.path.dirname(__file__), '..')))

from sputtertwin.physics.plasma import calculate_discharge_state

# === 1. Generate Synthetic Training Data ===

def generate_plasma_data(n_power=30, n_pressure=30):

powers = np.linspace(50, 500, n_power)

pressures = np.linspace(1, 20, n_pressure)

data = []

for W in powers:

for P in pressures:

ds = calculate_discharge_state(W, P)

data.append([W, P, 20.0, ds.voltage_v, ds.current_a,

ds.ion_flux, ds.electron_temp_ev, ds.plasma_density_m3])

return np.array(data)

# === 2. Define the PINN ===

class PlasmaPINN(nn.Module):

def __init__(self):

super().__init__()

self.net = nn.Sequential(

nn.Linear(3, 64), nn.Tanh(),

nn.Linear(64, 64), nn.Tanh(),

nn.Linear(64, 64), nn.Tanh(),

nn.Linear(64, 64), nn.Tanh(),

nn.Linear(64, 5),

)

def forward(self, x):

raw = self.net(x)

Vd = F.softplus(raw[:, 0:1])

Id = F.softplus(raw[:, 1:2])

Gi = F.softplus(raw[:, 2:3])

Te = 2.0 + 2.0 * torch.sigmoid(raw[:, 3:4])

ne = F.softplus(raw[:, 4:5])

return torch.cat([Vd, Id, Gi, Te, ne], dim=1)

# === 3. Loss Function ===

def pinn_loss(model, x_data, y_data, x_colloc, lambda_phys=0.1):

y_pred = model(x_data)

L_data = F.mse_loss(y_pred, y_data)

y_coll = model(x_colloc)

W = x_colloc[:, 0:1]

Vd = y_coll[:, 0:1]

Id = y_coll[:, 1:2]

L_power = F.mse_loss(W, Vd * Id)

return L_data + lambda_phys * L_power

# === 4. Training Loop ===

def train():

data = generate_plasma_data()

np.random.shuffle(data)

x_raw = data[:, :3]

y_raw = data[:, 3:]

x_mean, x_std = x_raw.mean(0), x_raw.std(0)

y_mean, y_std = y_raw.mean(0), y_raw.std(0)

x_norm = (x_raw - x_mean) / x_std

y_norm = (y_raw - y_mean) / y_std

x_train = torch.tensor(x_norm[:800], dtype=torch.float32)

y_train = torch.tensor(y_norm[:800], dtype=torch.float32)

x_colloc = torch.tensor(

np.column_stack([np.random.uniform(50, 500, 2000),

np.random.uniform(1, 20, 2000),

np.full(2000, 20.0)]),

dtype=torch.float32)

x_colloc = (x_colloc - torch.tensor(x_mean, dtype=torch.float32)) / \

torch.tensor(x_std, dtype=torch.float32)

model = PlasmaPINN()

optimizer = torch.optim.Adam(model.parameters(), lr=1e-3)

for epoch in range(5000):

optimizer.zero_grad()

loss = pinn_loss(model, x_train, y_train, x_colloc)

loss.backward()

optimizer.step()

if epoch % 500 == 0:

print(f"Epoch {epoch}: Loss = {loss.item():.6f}")

return model

if __name__ == '__main__':

model = train()

torch.save(model.state_dict(), 'plasma_pinn_cu.pt')

print("Training complete!")

## Directory Structure After Implementation

sputter_twin/

├── sputtertwin/

│   ├── physics/              # Existing analytical models (unchanged)

│   │   ├── plasma.py

│   │   ├── sputter_yield.py

│   │   ├── transport.py

│   │   └── deposition.py

│   ├── pinn/                 # NEW: PINN module

│   │   ├── __init__.py

│   │   ├── plasma_pinn.py

│   │   ├── yield_pinn.py

│   │   ├── transport_pinn.py

│   │   ├── deposition_pinn.py

│   │   ├── coupled_pinn.py

│   │   ├── losses.py

│   │   ├── data_loader.py

│   │   └── training.py

│   └── __init__.py

├── data/                     # NEW: Experimental data

│   ├── cu_experiments.csv

│   └── synthetic/

├── models/                   # NEW: Trained model checkpoints

├── tests/

│   ├── test_physics.py       # Existing (unchanged)

│   └── test_pinn.py          # NEW

├── scripts/

│   ├── generate_plots.py     # Existing

│   ├── train_pinn.py         # NEW

│   └── evaluate_pinn.py      # NEW

└── requirements.txt          # Updated with torch, pandas, etc.

# Summary & Key Takeaways

1. Our existing SputterTwin project is perfectly set up for PINNs — the analytical physics models become the physics loss terms that constrain the neural networks.

2. Start with Cu target material — we have well-defined material properties (TargetMaterial("Cu", 63.55, 29, 8.96, 3.49, 20.0, 1.60)) and the physics is well-understood.

3. Build sequentially — Plasma PINN → Yield PINN → Transport PINN → Deposition PINN → Coupled system.

4. Pre-train on synthetic data from our existing analytical models, then fine-tune on experimental measurements.

5. Multi-material is possible via material-conditioned inputs (adding M, Z, ρ, Us, Eth, Q as input features) or transfer learning. You do NOT need a completely separate model for each material.

6. The biggest value of PINNs for our project is: (a) Inverse problems — discovering unknown parameters from data, (b) Real-time prediction — millisecond inference, (c) Robust extrapolation — physics prevents unphysical predictions, (d) Data efficiency — learning from sparse data.

[IMPORTANT] Next Step: Start by implementing the minimal Plasma PINN example in Appendix (Chapter 16). This is a self-contained, runnable script that generates synthetic data from your existing calculate_discharge_state(), trains a PINN, and validates it. Once that works, extend to the other sub-processes.

— End of Document —